Department of Electrical and Computer Engineering

COEN 312                                                                               Oct. 16, 2008

Answer all Questions.

Exam Duration 1 hour

No books or papers are allowed.                       Examiner: Asim J. Al-Khalili

===============================================================

- Question  1

- Using Boolean algebra, minimize the following functions:

a) F1 (A,B,C,D) = ABCD + (A’BD)’ + ABC’D                       3 Marks

b) F2 (A,B,C,D) = AB (D + C’D ) + B ( A + A’CD)                3 Marks

- Question  2

Given   f(x,y,z) = xy +x z’ + yz

a) Implement f in NOR-NOR format                                         3 Marks

b) Implement f in AND-OR-INVERT format                            3 Marks

Obtain optimum implementation.

Question 3

Implement the following four Boolean expressions with three Half Adders:

F1(A,B,C) =  A⊕B⊕C                                                    1 Mark

F2( A,B,C) = A’BC + AB’C                                                  1 Marks

F3 (A,B,C) = ABC’ + ( A’ + B ‘ ) C                                      4 Marks

F4 (A,B,C) = ABC                                                                  1 Mark

Question 4

- Simplify the function F Using K-Map. Express the simplified function in SOP and POS

F(A,B,C,D) = Σ m(1,3,8,10,15) + d Σ m(0,2,9)             3 Marks

- For Function F below, find all the Prime Implicants and determine which ones are essential:

F(A,B,C,D) = Σ m( 1,3,6,7,8,9,12,13,14,15)                  3 Marks

SOLUTION

COEN312, OCT16 2008

Q1.

F1(A,B,C,D)=ABCD+(A'BD)'+ABC'D=ABCD+A+B'+D'+ABC'D=A+B'+D'

F1(A,B,C,D)=AB(D+C'D)+B(A+A'CD)=ABD+B(A+CD)=ABD+AB+BCD=AB+BCD

Q2.

a)

f(x,y,z)=xy+xz'+yz=xz'+yz

f in NOR-NOR format:

K-Map representation:

f(x,y,z)=(x+z)⋅(y+z')

Also, using Boolean minimization,

f(x,y,z)=xz'+yzf=(x+z).(y+z')f=(x+z)+(y+z')

b)

| f=; Z \ XY; 00; 01; 11; 10. 0; 0; 0; 1; 1. 1; 0; 1; 1; 0. | f '=; Z \ XY; 00; 01; 11; 10. 0; 1; 1; 0; 0. 1; 1; 0; 0; 1. |
| --- | --- |

f'(x,y,z)=xz+yz

Q3.

F2=(A'B+AB').C=(A⊕B).C

F3=ABC'+(A'+B')C=ABC'+(AB)'C=(AB)⊕C

Q4.

a)

| f= | CD\AB | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
|  | 00 | X | 0 | 0 | 1 |
|  | 01 | 1 | 0 | 0 | X |
|  | 11 | 1 | 0 | 1 | 0 |
|  | 10 | X | 0 | 0 | 1 |

SOP=m(0,1,2,3)+m(0,2,8,10)+m(13)=AB+BD+ABCD

POS=M(4,5,12,13)⋅M(4,6,12,14)⋅M(4,5,6,7)⋅M(9,11)=(A+B)(B+C)(B+D)(A+B+D)

b)

| m(12,13,14,15)=AB=NE f= | CD\AB | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
|  | 00 | 0 | 0 | 1 | 1 |
|  | 01 | 1 | 0 | 1 | 1 |
|  | 11 | 1 | 1 | 1 | 0 |
|  | 10 | 0 | 1 | 1 | 0 |

m(8,9,12,13)=AC=E

m(6,7,14,15)=BC=E

m(1,3)=ABD=E, m(3,7)=ACD=NE

E=Essential Prime Implicant

NE= Non-Essential Prime Implicant